Solar Pool Heating Controller

Technical Reference & Test Plan

Firmware v1.3.0  |  Raspberry Pi Pico W  |  MicroPython

| main.py v0.9.5 | controller/controller.py v0.9.8 | net/mqtt.py v0.5.1 | ota.py v0.1.2 |
| --- | --- | --- | --- |

# 1  System Overview

The Solar Pool Heating Controller runs on a Raspberry Pi Pico W and manages a pool circulation pump to capture solar heat from rooftop panels. It reads water temperatures at two points in the circuit (flow and return), computes the temperature delta, and runs the pump whenever the solar gain justifies it. Safety interlocks prevent the pump from running dry or the controller electronics from overheating.

Remote monitoring and control is provided via Adafruit IO over MQTT. The system can be operated automatically, manually boosted for a timed period, or placed into holiday mode from a dashboard. Firmware updates are delivered over-the-air (OTA) from a GitHub repository.

## 1.1  Hardware

| Item | Detail |
| --- | --- |
| Microcontroller | Raspberry Pi Pico W (RP2040 + CYW43439 WiFi) |
| Temperature sensors | DS18x20 on OneWire bus (GPIO 0) |
| Sensor labels | tFlow, tReturn, tAmbient, tEnclosure, tSolarPlate, tSolarRef |
| Water level | Capacitive sensor on GPIO 14 (HIGH = water present) |
| Pump relay | GPIO 16 (HIGH = pump ON) |
| Status LED | "LED" (Pico W onboard LED, active-high) |
| Manual button | GPIO 3, PULL_UP (LOW = pressed) |

## 1.2  Software Modules

| Module | Responsibility |
| --- | --- |
| main.py | Boot sequence and main loop. Owns sensor read cycle and OTA trigger. |
| state.py | Single shared dict visible to all modules. Fail-safe defaults. |
| config.py | All tunable constants and GPIO pin assignments. |
| controller/controller.py | Safety chain, solar logic, manual boost, periodic test, MQTT publish. |
| sensors/temperature.py | Non-blocking DS18x20 driver. Two-phase convert/read cycle. |
| sensors/water_level.py | Capacitive sensor with debounce and wet/dry confirmation delays. |
| sensors/button.py | Debounced edge-detecting button (single-fire per press). |
| net/__init__.py | Aggregates WiFi + MQTT. Owns watchdog restart callback and pre_restart_hook. |
| net/wifi.py | Non-blocking WiFi keepalive. Blocking connect at boot only. |
| net/mqtt.py | Publish/subscribe with rate limiting, bounded queue, watchdog. |
| net/feeds.py | Single source of truth for all Adafruit IO topic strings. |
| status_led.py | Non-blocking LED pattern driver. Priority-based pattern selection. |
| ota.py | OTA update manager. All-or-nothing apply with boot-time rollback. |
| utils.py | Bounded log buffer and MQTT flush helper. |

# 2  Boot Sequence

The boot sequence is deliberately ordered so that safety checks happen before anything else can fail.

┌─────────────────────────────────────────────────────┐

│                      BOOT                           │

│  1. verify_or_rollback()  ← OTA integrity check     │

│  2. Read manifest.json → state["fw_version"]        │

│  3. Sensors init + first read                        │

│  4. Network.connect() + NTP sync  (blocking)         │

│  5. Controller init + pre_restart_hook               │

└──────────────────────┬──────────────────────────────┘

│

┌──────────────────────▼──────────────────────────────┐

│                  MAIN LOOP (while True)             │

│                                                     │

│  Every 2 s ──► Read temperature sensors             │

│            ──► Read water level sensor              │

│                                                     │

│  Every tick ─► network.loop()  (WiFi + MQTT)        │

│            ──► controller.loop()                    │

│            ──► status_led.update()                    │

│            ──► gc.collect()                         │

│                                                     │

│  On flag ────► OTA check (pump off first)           │

└─────────────────────────────────────────────────────┘

| ⚠️ IMPORTANT | verify_or_rollback() is called before any other import. If a bad OTA update was applied, the system restores the previous firmware and reboots before any broken module is ever imported. |
| --- | --- |

After verify_or_rollback() passes, the boot sequence:

- Initialises all sensor objects (TemperatureSensor, WaterLevelSensor, Button)

- Reads manifest.json to populate state["fw_version"] (the bundle version reported in the debug feed)

- Performs one blocking sensor read so state is populated with real values before the first controller loop

- Connects WiFi and MQTT (blocking at boot — acceptable since we want connectivity before entering the loop)

- Syncs the Pico RTC via NTP (sets UK local time for correct core-hours comparisons)

- Constructs the Controller and wires network.pre_restart_hook (pump off before any watchdog reset); enters the non-blocking main loop

# 3  Controller Logic

The controller runs once per main loop iteration. Every subsystem within it is non-blocking. The loop is structured as a strict priority chain — higher-priority conditions always override lower ones.

controller.loop()

│

│

│

│

│

├─2─ _check_local_button()    ← poll physical button

│     Pressed? → set manual_override = True

│

├─3─ _run_safety_chain()      ← HIGHEST PRIORITY

│     │

│     ├─P1─ water_level_ok?      NO  → pump OFF (dry run protect)

│     │                               manual_override cleared

│     │                               RETURN (skip all logic)

│     │

│     ├─P2a─ tEnclosure present? NO  → pump OFF (sensor missing)

│     │                               RETURN

│     │

│     ├─P2b─ tEnclosure > 55°C?  YES → pump OFF (overtemp)

│     │                               manual_override cleared

│     │                               RETURN

│     │

│     └─P3─ tFlow/tReturn present? NO → pump OFF

│                                       RETURN

│

├─4─ _run_control_logic()     ← only if safety chain passed

│     │

│     ├─ manual_override?     YES → run timed boost (90 s)

│     │                            RETURN when active

│     │

│     ├─ core hours? (8–18)   NO  → pump OFF (sleep/holiday)

│     │  holiday mode?             RETURN

│     │

│     ├─ Solar delta logic

│     │   delta = tReturn – tFlow

│     │   delta >= 0.5°C AND pump off → pump ON

│     │   delta <  0.1°C AND pump on  → pump OFF

│     │   between thresholds          → no change (hysteresis)

│     │

│     └─ Periodic test

│         Every 600 s → run pump for 90 s to circulate water

│         so tFlow/tReturn readings reflect actual roof temp

│

└─5─ _check_publish()  ← 30 s (core hours) / 300 s (off-hours)

## 3.1  Safety Chain

The safety chain is the most important part of the system. It runs on every loop iteration regardless of mode, and its decisions cannot be overridden by MQTT commands, button presses, or any other control input.

Priority  Condition                  Action

────────  ─────────────────────────  ───────────────────────────────

1 (HIGH)  Water level LOW            Pump OFF, dry_run_protect = True

(capacitive sensor)        manual_override cleared

────────  ─────────────────────────  ───────────────────────────────

2a        tEnclosure sensor MISSING  Pump OFF, enclosure_sensor_missing

manual_override cleared

────────  ─────────────────────────  ───────────────────────────────

2b        tEnclosure > 55°C          Pump OFF, overtemp_shutdown = True

manual_override cleared

────────  ─────────────────────────  ───────────────────────────────

3         tFlow or tReturn MISSING   Pump OFF

(LED: sensor_missing pattern)

────────  ─────────────────────────  ───────────────────────────────

4         manual_override = True     Pump ON for 90 s

(MQTT or button)           then falls through to auto

────────  ─────────────────────────  ───────────────────────────────

5 (LOW)   Solar delta / hours/       Normal automatic control

holiday / test run

| 🔴 SAFETY NOTE | When water_level_ok is False, the pump is stopped immediately with urgent=True, which bypasses the MQTT rate limiter so the safety stop is published without delay. |
| --- | --- |

## 3.2  Solar Delta Logic

When the safety chain passes and the system is within core hours (08:00–18:00), the controller compares tReturn and tFlow to determine whether solar heating is occurring.

| Condition | Action |
| --- | --- |
| delta = tReturn − tFlow | The temperature difference between roof return and pool flow |
| delta >= 0.5°C (pump off) | Turn pump ON — solar gain is sufficient |
| delta < 0.1°C (pump on) | Turn pump OFF — gain insufficient to justify running |
| 0.1°C ≤ delta < 0.5°C | No change — hysteresis band prevents rapid cycling |

| ℹ️ NOTE | auto_wants_pump is initialised to the current pump state each loop. This means the hysteresis is implemented both by the threshold gap AND by the state initialisation — they work together. |
| --- | --- |

## 3.3  Manual Boost

A manual boost runs the pump for a fixed duration (default 90 seconds) regardless of solar delta or time of day. It is cancelled immediately by any safety condition.

- Triggered by: physical button press OR MQTT mode 1

- Duration: manual_override_duration (config, default 90 s)

- The boost timer starts on the first loop after the flag is set

- The test timer is reset when a boost starts, so a test doesn't fire immediately after

- Holiday mode (MQTT mode 2) cancels any active boost

## 3.4  Periodic Test Run

The temperature sensors rely on water flowing through the circuit to give meaningful readings. When the pump is off, tFlow and tReturn stagnate and no longer reflect actual roof conditions. To get an accurate picture, the pump runs briefly every 10 minutes (configurable) during core hours. This circulates water through the panels so the sensor readings update, allowing the solar delta logic to make a correct heating decision. The test duration (default 90 s) is long enough for the readings to stabilise.

- The test interval is measured from the end of the previous test, not the start

- If a safety condition fires mid-test the pump stops immediately, but test_running is not cleared — the test terminates on the first loop after the condition clears

- After the test completes, solar delta logic is re-evaluated immediately

From v1.3.0 the test is also gated on solar irradiance. A test only produces a useful reading if the panel is actually warmer than the water sitting in the circuit; if it is not, the test ends with the pump switching straight back off and the 90 s run is wasted. The gate signal is plate_pool_delta (tSolarPlate − tFlow), evaluated with the pump off immediately before a scheduled test. The threshold was derived from several months of logged test outcomes.

Gate signal: plate_pool_delta (tSolarPlate − tFlow), computed every loop and included in every debug payload so the threshold can be verified from the logs

A scheduled test runs only if plate_pool_delta ≥ gate_threshold (config, default 0.5 °C)

Once a test slot is due the gate is re-evaluated every loop, so a test fires as soon as the panel warms rather than waiting for the next interval boundary

FALLBACK: if gate_fallback_interval (config, default 3600 s) has elapsed since the last test of any outcome, the test runs regardless of the gate

FAIL-SAFE: if tSolarPlate or tFlow is unavailable the gate is treated as open and behaviour reverts to the timer-only schedule. A sensor fault can never prevent heating. The safety interlocks are unaffected — they run earlier in the loop and have already stopped the pump

gate_enabled = False in config disables the gate entirely, restoring pre-v1.3.0 behaviour without a code change

A skipped test publishes one debug record with pump_reason "test skipped (gate X.XC)" per stagnation period — not per slot, so the logging cadence is unchanged

The gate delays tests only. If the solar delta itself clears delta_threshold_high, heating starts regardless of the gate

## 3.5  Holiday Mode

When MQTT mode 2 is received, manual_disabled is set to True. The pump will not run automatically. A manual boost also cannot be triggered while in holiday mode. The mode is held in RAM only and does NOT persist across a restart — after a reboot the controller returns to automatic operation. Send MQTT mode 0 to resume automatic operation at any time.

# 4  Status LED

The status LED provides a visual indication of system health without requiring a serial connection. Patterns are selected by priority — only the highest-priority active condition is shown. The LED runs entirely from the shared state dict and never blocks.

Pattern         Sequence              Trigger condition

──────────────  ────────────────────  ───────────────────────────────────

critical        1 s ON / 1 s OFF      state[critical_error] = True

(slow steady blink)   (unrecoverable software error)

──────────────  ────────────────────  ───────────────────────────────────

sensor_missing  1.5 s ON / 0.1 s OFF  Any sensor disconnected OR

(long on, blip off)   enclosure sensor absent

──────────────  ────────────────────  ───────────────────────────────────

mqtt_offline    0.25 s ON / 0.25 s    MQTT not connected

OFF (fast flash)      (WiFi may still be up)

──────────────  ────────────────────  ───────────────────────────────────

ok              0.2 s ON / 2 s OFF    Everything healthy

(heartbeat)

| ℹ️ NOTE | enclosure_sensor_missing shows the sensor_missing pattern, not overtemp_shutdown. This is intentional — a missing enclosure sensor is a sensor fault, not a temperature alarm. |
| --- | --- |

# 5  Network Architecture

┌─────────────┐     WiFi (WPA2)      ┌──────────────┐

│   Pico W    │ ──────────────────►  │   Router     │

│             │                      └──────┬───────┘

│  WiFiManager│                             │ Internet

│  MQTTManager│ ◄──── MQTT (port 1883) ────►│

│             │                      ┌──────▼───────┐

│  feeds.py   │                      │ Adafruit IO  │

└─────────────┘                      │  io.adafruit │

│  .com        │

Subscribed feeds (inbound):          └──────────────┘

pump_override  ← mode 0/1/2 from dashboard

ota_trigger    ← '1' triggers OTA check

Published feeds (outbound):

pump_state     tFlow                tReturn

tAmbient       delta-t-flow-return  delta-irradiance

debug (full state JSON)

## 5.1  WiFi

- Blocking connect at boot (up to 10 retries × 2 s = 20 s max)

- Non-blocking reconnect in loop() — one attempt per 10 s, never sleeps

- MQTT reconnect waits for WiFi to be confirmed up before attempting

## 5.2  MQTT

- Inbound feeds subscribed on connect: pump_override, ota_trigger

- Publish rate limit: 10 s per topic (prevents Adafruit IO throttling)

- urgent=True bypasses rate limit for safety-critical state changes

- Offline queue: bounded at 20 messages, oldest dropped when full

- Queue flushed on reconnect

- Watchdog: triggers system restart after 3 × watchdog_interval (3 min) of sustained outage

## 5.3  MQTT Command Reference

| Command | Effect |
| --- | --- |
| pump_override = 0 | Cancel holiday mode. Return to automatic. (Boost timer unaffected.) |
| pump_override = 1 | Trigger manual boost (90 s). One-shot — dashboard resets to 0 automatically. |
| pump_override = 2 | Enable holiday mode. System stays off until mode 0 received. |
| ota_trigger = 1 | Trigger OTA firmware check from GitHub manifest. |

# 6  OTA Update System

MQTT ota_trigger = '1'

│

▼

state[ota_pending] = True

│

main.py detects flag

│

pump OFF (urgent)   ← safety first

│

fetch manifest.json from GitHub

│

compare __version__ strings

│

┌──────┴──────────┐

│ Files to update?│

└──────┬──────────┘

NO   │  YES

│    ▼

│  Download all → .new files

│    │

│  All OK?  ── NO ──► delete .new files, return (safe)

│    │

│   YES

│    │

│  Verify each .new with compile()

│    │

│  All OK?  ── NO ──► delete .new files, return (safe)

│    │

│   YES

│    │

│  Apply: .new → file, old → .bak

│  (ota.py applied last)

│    │

│  machine.reset()

│

▼

return (no update needed)

── NEXT BOOT ──

verify_or_rollback()

│

Check CRITICAL_FILES compile OK

│

OK?  ── NO ──► restore .bak files → reboot

│

YES

│

Delete .bak files (free flash)

│

Normal boot continues

## 6.1  Manifest Format

The manifest.json is hosted at secrets.OTA_MANIFEST_URL (a GitHub raw content URL). Each file entry maps a path to its expected version string. The version string must match __version__ in the file exactly.

{

"version": "2.1.0",

"base_url": "https://raw.githubusercontent.com/USER/REPO/main/firmware",

"files": {

"main.py": "2.1.0",

"controller/controller.py": "0.6.0",

...

}

}

## 6.2  Update Workflow

When pushing a firmware update:

- Bump __version__ in the changed source file(s)

- Update the matching entry in manifest.json

- Push both to GitHub

- Send ota_trigger = 1 via MQTT

- The Pico downloads only changed files, verifies them, applies atomically, and reboots

- On the next boot, verify_or_rollback() confirms the update is clean or restores .bak files

## 6.3  Rollback

If verify_or_rollback() detects a broken critical file after an OTA update, it restores all .bak files and reboots. This is fully automatic — no physical access to the Pico is required. Critical files checked: main.py, state.py, ota.py, controller/controller.py, net/__init__.py, sensors/__init__.py.

# 7  State Dictionary Reference

All modules share a single dict defined in state.py. The controller writes to it each loop; MQTT publishing reads from it. Safety-critical booleans default to the restrictive (safe) value.

| Key | Default | Description |
| --- | --- | --- |
| pump_on | False | Current pump relay state |
| pump_reason | "" | Human-readable reason for current pump state |
| manual_override | False | Timed boost active (button or MQTT mode 1) |
| manual_disabled | False | Holiday mode — system stays off |
| test_running | False | True while periodic test run is active |
| last_pump_run | None | Timestamp of last pump activation |
| time_to_next_test | 0 | Seconds until next periodic test (computed) |
| delta_t_flow_return | None | tReturn − tFlow (°C), computed each loop; None when sensors absent |
| delta_irradiance | None | tSolarPlate − tSolarRef (°C), computed each loop. Solar irradiance proxy — monitoring only, gates nothing |
| plate_pool_delta | None | tSolarPlate − tFlow (°C), computed each loop. Gate signal for the periodic test |
| test_gate_open | True | Result of the last irradiance gate evaluation. FAIL-SAFE default is True (open = test on the timer alone) |
| last_test_ts | None | Timestamp when last periodic test completed |
| temps | {} | Dict of label → temperature °C |
| water_level_ok | False | FAIL-SAFE: False until sensor confirms wet |
| sensors_ok | False | FAIL-SAFE: False until all sensors report |
| disconnected_sensors | [] | Labels of sensors that failed last read |
| wifi_connected | False | WiFi connection state |
| mqtt_connected | False | MQTT connection state |
| time_synced | False | True after first successful NTP sync |
| last_ntp_sync | None | Timestamp of last successful NTP sync |
| overtemp_shutdown | False | Enclosure temperature exceeded limit |
| dry_run_protect | False | Pump stopped due to low water level |
| enclosure_sensor_missing | False | Enclosure sensor absent (distinct from overtemp) |
| critical_error | False | Unrecoverable software error → LED critical pattern |
| ota_pending | False | Set True by MQTT to trigger OTA check |
| fw_version | "unknown" | Bundle version string from manifest.json, populated at boot; published in debug feed |
| last_error | None | String message from last caught exception |

# 8  GUI Test Plan

All tests are run using the desktop simulation harness (test_main_controller_gui.py) before deploying to the Pico. The GUI provides full control over sensor inputs, WiFi state, time mode, and manual controls. Use the state dump panel to verify internal state after each test.

| 📋 SETUP | Run: python test_main_controller_gui.py Verify the GUI opens, the state dump panel populates, and the LED shows the 'ok' heartbeat pattern within a few seconds. |
| --- | --- |

## 8.1  Boot and Initial State

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T01 — Fail-safe defaults Verify state starts safely before sensors are read. | Launch GUI. Observe state dump immediately on load. | water_level_ok = False sensors_ok = False pump_on = False mqtt_connected = False |  |
| T02 — Initial sensor read Verify state is populated from GUI sliders on first tick. | Set all temp sliders to 25°C. Water Present = ticked. | temps contains tFlow, tReturn, tAmbient, tEnclosure water_level_ok = True sensors_ok = True |  |
| T03 — LED starts OK Verify LED shows heartbeat when all inputs are healthy. | All sliders 25°C, Water Present, WiFi ticked. | LED pattern = 'ok' LED blinks briefly every ~2 s |  |

## 8.2  Safety Chain — Water Level

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T04 — Dry run protect Verify pump stops immediately when water level drops. | Set time to Day. Raise tReturn to 30°C, tFlow to 25°C (delta = 5°C, pump should be ON). Then untick Water Present. | pump_on → False pump_reason = 'dry run protect' dry_run_protect = True LED pattern = 'sensor_missing' |  |
| T05 — Boost cannot override dry run Verify manual boost is cancelled by water loss. | Pump running on boost. Untick Water Present. | pump_on → False immediately manual_override → False dry_run_protect = True |  |
| T06 — Recovery from dry run Verify pump resumes when water returns. | From T04 state. Re-tick Water Present. | dry_run_protect → False Pump resumes normal logic within 1–2 ticks LED returns to 'ok' |  |

## 8.3  Safety Chain — Enclosure Temperature

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T07 — Enclosure sensor missing Verify distinct flag when tEnclosure sensor absent. | Tick 'Disable' next to tEnclosure slider. | pump_on = False pump_reason = 'enclosure sensor missing' enclosure_sensor_missing = True overtemp_shutdown = False LED = 'sensor_missing' |  |
| T08 — Overtemp shutdown Verify pump stops when enclosure is too hot. | Slide tEnclosure above 55°C. | pump_on = False pump_reason = 'enclosure overtemp' overtemp_shutdown = True enclosure_sensor_missing = False |  |
| T09 — Overtemp recovery Verify pump resumes when enclosure cools. | From T08. Slide tEnclosure back to 30°C. | overtemp_shutdown → False Pump resumes normal logic LED returns to 'ok' |  |

## 8.4  Safety Chain — Flow/Return Sensors

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T10 — Flow sensor missing Verify pump stops when tFlow is absent. | Tick 'Disable' on tFlow. | pump_on = False pump_reason = 'flow/return sensor missing' LED = 'sensor_missing' |  |
| T11 — Return sensor missing Verify pump stops when tReturn is absent. | Restore tFlow. Tick 'Disable' on tReturn. | pump_on = False LED = 'sensor_missing' |  |
| T12 — Recovery Verify auto logic resumes when both sensors return. | Re-enable both sensors. tReturn = 30, tFlow = 25. | sensors_ok → True Pump resumes based on delta LED → 'ok' |  |

## 8.5  Solar Delta Logic

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T13 — Pump ON at high threshold Verify pump starts when delta >= 0.5°C. | Time = Day. All sensors healthy. tFlow = 25.0, tReturn = 25.4 (delta = 0.4, below threshold). Then raise tReturn to 25.5 (delta = 0.5). | At delta 0.4: pump_on = False At delta 0.5: pump_on = True pump_reason contains 'solar heating' |  |
| T14 — Pump OFF at low threshold Verify pump stops when delta < 0.1°C (hysteresis). | Pump running (T13). Lower tReturn to 25.1 (delta = 0.1). Then to 25.09 (delta = 0.09). | At delta 0.1: pump_on = True (hysteresis holds) At delta <0.1: pump_on = False pump_reason contains 'insufficient gain' |  |
| T15 — No pump outside core hours Verify pump does not start at night even with good delta. | Set time to Night. tReturn = 30, tFlow = 25 (delta = 5°C). | pump_on = False pump_reason contains 'sleep' |  |
| T16 — Pump stops at night Verify running pump stops when night mode selected. | Start with pump running in Day mode. Switch time to Night. | pump_on → False within 1–2 ticks pump_reason contains 'sleep' |  |

## 8.6  Manual Boost

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T17 — Boost in day mode Verify boost runs pump during core hours. | Time = Day. Delta below threshold (pump off). Tick Manual Boost checkbox. | pump_on = True pump_reason contains 'manual boost' manual_override = True |  |
| T18 — Boost outside core hours Verify boost overrides sleep mode. | Time = Night. Tick Manual Boost. | pump_on = True despite night mode pump_reason = 'manual boost' |  |
| T19 — Boost expires Verify boost stops after duration and falls through to auto. | Boost active (T17). Wait 90 s (or set manual_override_duration to 5 s in config for testing). | manual_override → False pump_on determined by solar delta logic Checkbox unchecks itself |  |
| T20 — Safety overrides boost Verify dry run cancels active boost. | Boost active. Untick Water Present. | pump_on → False manual_override → False dry_run_protect = True |  |

## 8.7  Periodic Test Run

| 💡 TIP | Set pump_test_interval to 15, pump_test_duration to 5 and gate_fallback_interval to 60 in config.py for faster testing. Restore to 600 / 90 / 3600 before deploying. |
| --- | --- |

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T21 — Test fires on schedule Verify periodic test starts after interval. | Time = Day. Delta below threshold (pump off). tSolarPlate at least 0.6 °C above tFlow so the gate is open. Use short test_interval. Watch time_to_next_test count down. | When time_to_next_test reaches 0: test_running = True pump_on = True pump_reason = 'periodic test' |  |
| T22 — Test stops after duration Verify test ends and logic re-evaluates. | Test running (T21). Wait for test_duration to elapse. | test_running → False Pump state determined by solar delta time_to_next_test resets to full interval |  |
| T23 — Test does not fire at night Verify test is not triggered outside core hours. | Set time to Night. Wait well past test_interval. | test_running = False throughout Pump stays OFF |  |
| T36 — Gate allows test Verify an open gate does not change existing behaviour. | Time = Day. tSolarPlate ≥ tFlow + 0.6. Short test_interval. | At countdown 0: test_running = True pump_reason = 'periodic test' test_gate_open = True |  |
| T37 — Gate skips test Verify a cold panel suppresses the test. | Time = Day. tSolarPlate within 0.5 °C of tFlow. Short test_interval, long gate_fallback_interval. | Countdown reaches 0 but no test runs. test_running = False test_gate_open = False pump_reason = 'test skipped (gate X.XC)' once, then reverts to 'waiting for solar gain' Pump stays OFF 'Next test' readout shows '— gated' |  |
| T38 — Gate opens mid-wait Verify the test fires as soon as the panel warms. | Continue from T37. Raise tSolarPlate above tFlow + 0.5. | Test starts within a tick or two — it does NOT wait for the next interval boundary |  |
| T39 — Fallback forces a test Verify the gate cannot block tests indefinitely. | Set gate_fallback_interval to 60. Keep tSolarPlate below tFlow + 0.5. Wait 60 s past the last test. | Test runs anyway. Console logs 'Starting periodic test (gate fallback)' test_gate_open remains False |  |
| T40 — Fail-safe: plate sensor missing Verify a sensor fault never blocks heating. | Disable tSolarPlate with its checkbox. Wait for the test interval. | plate_pool_delta = None test_gate_open = True Test runs on the timer exactly as before the gate existed |  |
| T41 — Gate disabled Verify the config kill switch. | Set gate_enabled = False in config.py. tSolarPlate below tFlow. | Tests run purely on the timer. test_gate_open stays True |  |

## 8.8  Network and MQTT

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T24 — WiFi disconnect Verify LED reflects MQTT loss when WiFi drops. | System running normally. Untick WiFi Connected. | mqtt_connected → False LED pattern → 'mqtt_offline' (fast flash) Pump logic continues unaffected |  |
| T25 — WiFi reconnect Verify MQTT reconnects when WiFi returns. | From T24. Re-tick WiFi Connected. Wait up to 15 s. | mqtt_connected → True LED → 'ok' Queued messages flushed |  |
| T26 — Pump state survives WiFi loss Verify safety logic is not affected by network state. | Good delta (pump ON). Untick WiFi. Wait 30 s. | pump_on remains True (or changes based on delta only) No safety flags set by network loss |  |

## 8.9  LED Patterns

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T27 — OK heartbeat Verify normal operating pattern. | All sensors healthy. WiFi and MQTT connected. | LED pattern = 'ok' Brief blink every ~2 s |  |
| T28 — sensor_missing pattern Verify correct pattern on sensor fault. | Disable any sensor. | LED pattern = 'sensor_missing' Long on / brief off cycle |  |
| T29 — mqtt_offline pattern Verify pattern when MQTT disconnected. | All sensors healthy. Untick WiFi. | LED pattern = 'mqtt_offline' Rapid flash |  |
| T30 — critical priority overrides sensor Verify critical beats sensor_missing. | Disable a sensor (sensor_missing). Then set state[critical_error]=True via state dump. | LED pattern → 'critical' Slow 1s on/off blink |  |
| T31 — sensor_missing priority over mqtt Verify sensor_missing beats mqtt_offline. | Untick WiFi (mqtt_offline). Then disable a sensor. | LED pattern → 'sensor_missing' (higher priority) |  |

## 8.10  Fake ROM / Unknown Sensor

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T32 — Unknown ROM warning Verify unknown ROM is handled gracefully. | Tick 'Use Fake ROMs'. This adds ROM '11111111111111' which is not in rom_to_label. | Console shows: ⚠️ Unknown ROM ... detected! System continues operating Unknown ROM appears in disconnected_sensors |  |
| T33 — Known ROMs still work Verify real sensors still read correctly with fake ROMs enabled. | Fake ROMs ticked. Adjust tFlow and tReturn sliders. | temps dict still contains tFlow, tReturn, tEnclosure, tAmbient Solar delta logic operates normally |  |

## 8.11  Simulated Restart

| Test / Steps | Setup | Expected Result | Pass/Fail |
| --- | --- | --- | --- |
| T34 — Watchdog restart Verify system recovers cleanly from a simulated restart. | System running. Trigger soft restart via the restart callback (MQTT watchdog fires after 3 min with WiFi off). | State resets to fail-safe defaults water_level_ok = False sensors_ok = False Network reconnects System resumes normal operation |  |
| T35 — Pump off before restart Verify pump is turned off before state reset. | Pump running. Trigger restart. | pump_on → False before state is cleared pump_reason = 'watchdog restart' |  |

# 9  Pre-Deployment Checklist

Before copying firmware to the Pico, confirm each item:

| ☐ | All GUI tests T01–T41 passed |
| --- | --- |
| ☐ | config.py: pump_test_interval restored to 600, pump_test_duration to 90, gate_fallback_interval to 3600 |
| ☐ | config.py: delta_threshold_high and low set to correct values for your installation |
| ☐ | config.py: core_start_hour and core_end_hour match your local solar window |
| ☐ | config.py: max_enclosure_temp appropriate for your enclosure (default 55°C) |
| ☐ | secrets.py: WIFI_SSID, WIFI_PASSWORD, AIO_USERNAME, AIO_KEY populated |
| ☐ | secrets.py: OTA_MANIFEST_URL points to your GitHub raw manifest.json |
| ☐ | manifest.json pushed to GitHub with correct version strings |
| ☐ | config.py: gate_enabled, gate_threshold and gate_fallback_interval reviewed for your installation |
| ☐ | DS18x20 ROM IDs verified against config.py rom_to_label |
| ☐ | Water level sensor tested wet and dry before installation |
| ☐ | Pump relay wired to GPIO 16, verified with a test run |
| ☐ | Status LED uses Pico W onboard LED (PIN_LED = "LED") — verify heartbeat blink at boot |
| ☐ | Manual boost button wired to GPIO 3 with pull-up confirmed |

Solar Pool Heating Controller  |  v1.3.0  |  Generated by Claude